RetroChallenge 2021/10 competition entry: Using the Commodore 64 to control a remote control car.

Description:

The Commodore 64 was released in 1982 and quickly became the best selling computer of all time. Many users treated the system as a games controller, but it was a real computer and could also be used as a microcontroller, using it’s ports like the User Port.

In this project I wanted to use this retro computer to control a remote control (RC) car. Using an inexpensive RC car and re-fitting it with an Arduino to allow connection to a radio transceiver (NRF24L01). On the Commodore 64 side I decided to use RS232, which on the C64 needs an adapter to bring it up to the voltages needed. The adapter released by Commodore was called the VIC-1011A.

Control of the RC car can be via a joystick and recorded (to a floppy disk) for later autonomous 'play back' by the C64. The C64 will use RS232 to a microcontroller with a wireless transceiver that can connect to the RC car. Commodore 64 control on an RC car By Steve Smit Started in September 2021

The code on the Arduinos used in this project is written using the IDE in the standard C++. The code on the Commodore 64 will initially be in BASIC, but will likely be re-written in either Forth (DurexForth) or assembler.

22nd September 2021 : First piece of progress, get C64 to send bytes representing the directions of the joystick by RS232 to a transceiver to a receiver to confirm 'commands' can be sent. Code on C64 written in DurexForth: https://youtu.be/1RyrSl5RLjI

I purchased 1/14 scale RC ‘Monster Truck’ off-road 2WD car for just under AUD$40.

I first tried to ‘hack’ the controller to see if perhaps this might mean I could leave the electronics in the car unmodified. This initially worked!:

https://youtu.be/TNTKXlKmA1Y

But, for no apparent reason, the ‘steering’ motor, while making a straining noise, now doesn’t seem to have the torque to turn the car anymore. Hacking the controller wasn’t ideal in any case, as I wanted to have more control via the Commodore 64. I decided to open the car up, firstly to see if I could address the steering motor issue, but also to replace the motor controller with a L298N dual h-bridge motor controller module and hook up an Arduino to do some testing.

Even directly connecting to the ‘steering’ motor didn’t fix the lack of movement issue. I have ordered a few replacement DC motors as an option to fix the issue. An alternative is to replace the steering mechanism with a servo motor. Here is how the current DC motor controls the steering:

While I wait for replacement items, I better get onto the code on the C64. I decided that the opening screen should offer the user 4 choices:

F1 = Operate RC car without recording

F3 = Operate RC car with recording

F5 = Show saved sequences

F7 = “Play” current sequence in memory           , or something like this

I realized that I didn’t know what byte is returned if someone presses the Function keys (F1, F3, etc.) so I wrote a quick basic program to get the values:

From this I see that F1 = 133, F3 = 134, F5 = 136 and F7 = 137.

For the joystick movements, I decided that each direction and the fire button be given a value that would be sent as a command to the RC car.

On a Commodore 64 the joystick in port 1 can be read by reading the contents of 56321 ($DC01). In BASIC this would be PEEK(56321).

I have chosen to write my code in assembler, so this is LDA $DC01

The joystick default is all 1s, i.e. 255 or $FF

Buttons (fire or a direction) pull bits low in the byte. Fire on it’s own is then = 239 or $EF

Up = 254, Down 253, Left 251, Right 247,

Up&Left 250, Up&Right 246, Down&Left 249,

and Down&Right 245. I then convert these to the commands in the diagram to the left.

27 September 2021 – Assembly programming

I’m still not sure what language to use on the Commodore 64, but I know that Assembly has the maximum level of control and speed, so l will make a start and see how I go. I’m using an IDE called CBM Prg Studio to enter the assembly that compiles the machine code:

Well, not the flashiest of menus, but it’s a start.

28 September 2021 – car steering motor repair

The replacement motor I ordered arrived and it fits perfectly and works! The Arduino sketch I used to test the car just spins the rear motor forward then backward, then confirms the front steering can be done. https://youtu.be/1il9svvlH1M

I still have a lot of coding to do on the Commodore 64, which I am persisting with Assembly language.

29 September 2021 – More machine coding

Now that I have a reasonable menu displayed, I need read the keyboard and act if one of the Function Keys is pressed.

1 October 2021

As the car has limited space, I decided to see if I could use an Arduino Nano in the car instead of the UNO. My first test was to see if I can send the commands for left, centre and right, including using a small servo to show this in action, and it worked! https://youtu.be/7N0zTxdJpyA

2nd October 2021 – Testing Nano in car with radio transceiver connected

After changing the Arduino UNO to an Arduino Nano, I did another test to confirm this works in the RC car. https://youtu.be/uOMSnjM_ak8 . I’m pleased to see that it worked fine.

3rd October 2021 – Test change from L298N to TB6612FNG

I also decided that the L298N is a rather bulky device and loses around 1.4v from the battery, that I should see if I can use the more efficient and smaller TB6612FNG H-Bridge instead.

Of course, this also needed to be tested https://youtu.be/IEmlqmNZZGI

I then updated both the Arduino sketch and C64 programs to test also the forward and reverse control. I’m pleased to see that this works too! https://youtu.be/RIUS3Q_FON8

4th October 2021 – Code on Nano (in car) updated to support all directions

While my initial test for forward, reverse, left and right worked independently of each other, I also need to be able to support both moving forward while turning, as per the diagonal directions of joystick. I also wanted a way to avoid full power going to the front steering motor when it’s reached the end of travel. This got more complicated that I anticipated, as I need to use millisecond timers and keep track of duration of time during turning as well as when already in fully locked turned position where I wanted to reduce power (down from 250 to 150 for instance).

I used a flag I called TTFlag (which stood for Turning Time Flag) which would be false when the steering motor was in the centre position. If the flag wasn’t false, it meant that the steering motor was already turning under full power, so I needed to also read a variable that was measuring time in milliseconds to know when to drop power to the steering motor from full to hold (i.e. 250 down to 150). I am pleased to report that I succeeded in getting this sorted out ( https://youtu.be/m04mfECTdz8 ). Now to re-charge the battery, secure the electronics and aerial and replace the top of the RC car, and give it a test on the floor.

5th October 2021 – First floor test (without securing the electronics or adding the top of the RC car)

OK, the RC car is working (sort of). All the controls are being properly interpreted, but there appears to be an excess of current for the motor controller. Fortunately, the motor controller has over-current/heating protection but this is resulting in jerky movement after less than 30 seconds of use. https://youtu.be/hHUsokOsonM . What I can be happy about is that all the other elements outside of the Commodore 64 seem to be working, so it’s just the code on the Commodore 64 that needs to be done now.

6th October 2021 – Coding on the Commodore 64

I decided to write my code in assembler. The Commodore 64 uses a MOS 6510 processor, which is essentially the same as the famous 6502 used in other systems like the Apple II, Atari 2600, BBC Micro and Nintendo Entertainment System, just to name a few. Assembler is more challenging, but the reward is that the code is small and fast. I used an IDE (Integrated Development Environment) called CBM prg Studio (https://www.ajordison.co.uk/), written by Arthur Jordison.

The program so far sets up the RS232 port at 1200 baud, displays the main menu and F1 (Operate RX car without recording) is functional https://youtu.be/LmQ5uK1qb5w

October 7th 2021 – Testing assembler to control RC car without recording

I worked on and completed a new assembler code routine called “RC CAR.ASM”. The code now waits until there is activity on the joystick before sending the appropriate command out via RS232. On the screen is also shown characters representing the directions the user is moving the joystick. For this function, exit is by pressing the fire button. https://youtu.be/aGeefbiyoig

Now I need to move on to writing code that will store the sequence, including the timing between each movement of the joystick. The code will be very similar to above, except that I will allocate the fire button as start and stop of the sequence recording. To exit I will use something like the space bar. This next coding will be the most challenging, and I still have the ‘play-back’ function code to write after that (which shouldn’t be as difficult, I hope).

October 8th 2021 – Starting to write the ‘function’ that will allow a sequence to be recorded

The Commodore 64 has a jiffy clock that ‘ticks’ every 16.67 milliseconds (60 per minute). After referring to the trusty Commodore 64 programmers reference guide, I see that the Kernal (think operating system) of the C64 after reset, increments from 0 the number of jiffies with the current count stored in most significant byte (of a 3 byte sequence) at $A0, $A1 and $A2. I wrote a small basic program just to confirm this indeed can be read to check the number of jiffies after starting.

The Jiffy clock can be reset by calling SETTIM ($FFDB) with the Accumulator, X and Y registers set to 0. Then when a change occurs (i.e. joystick movement), the values in the Zero Page address $A1 and $A2 contains the number of jiffies since last reset. Two bytes of jiffies will be sufficient for this project, as this equates to over 18 minutes maximum gap between changes to the RC car’s last ‘command’, which is way more than needed. I was tempted to just use one byte, but that only allows for 4.25 seconds which won’t be realistic for some driving sequences (i.e. someone could want to have the car drive forward for 10 seconds before stopping).

October 10th 2021 – Getting heavy into 6510 machine code programming (Assembler)

I have now written routines for starting and stopping recording of RC car sequences. A new routine that not only sends the command byte to the RC car (in real time) but also saves the time since the last command. I have also commenced writing code that will prompt for a filename to save the current sequence (which is triggered when the user hits the fire button for the 2nd time. The first time is to start recording [note to self, I may need to watch out for ‘button bounce’]).

; Send Byte X without recording

; Sub-routine to send a byte via RS232

SndByt  ldx #$02        ; Set device

jsr chkout      ; as output

lda X_Var       ; Command to be sent to RC car

jsr chrout      ; send the byte

ldx #3          ; The screen

jsr chkout

rts

; Send Byte X with recording

; Sub-routine to send a byte via RS232

SndBytR lda SeqPos      ; What position are we at

ldx #3          ; this needs to be multiplied by 3

jsr multi       ; Result acc = Hi Byte, x = Lo byte

stx RCarray     ; store in Lo byte

clc

adc #$40

sta RCarray+1   ; Hi byte order

ldy #0          ; initial offset from where RCarray is pointing

lda $a1         ; High Byte of Jiffies since last Reset

sta (RCarray),y  ; Store in the RC array ($4000 + SeqPos*3)

iny

lda $a2         ; Low Byte of Jiffies

sta (RCarray),y

iny

lda X_Var       ; load the command byte to be sent and stored

sta (RCarray),y

ldx #$02        ; Set device

jsr chkout      ; as output

lda X_Var       ; Command to be sent to RC car

jsr chrout

ldx #3          ; The screen

jsr chkout

inc SeqPos

jsr Reset       ; reset the jiffy clock timer

rts

October 11th 2021 – Added ability to prompt for a filename to be given to save a sequence file

; Get file name

; stores filename text at filetxt = $3f6c up to 18 characters

; will need to support backspace to delete back to fix file name error

; stores filename length in filnaml = $65

getflnm lda #0          ; Use filnaml to store filename length

sta filnaml     ; starting at 0

sta BLNSW       ; make sure cursor is flashing

@loop   jsr chrin       ; read keyboard buffer

beq @loop       ; loop if no character entered

cmp #13         ; Has a CR been pressed

beq @end        ; If so go end

cmp #148        ; check if a backspace (del) has been pressed

bne @cont       ; if not then jmp ahead to add character to filename

lda filnaml

beq @loop       ; if filename length is 0 then we can't backspace

dec filnaml

jmp @loop

@cont   ldx filnaml     ; load x as offset for storing filename

sta filetxt,x   ; Store each charater at filetxt

inc filnaml     ; Increment filnaml for each character stored

jmp @loop       ; return back to get more char or a CR

@end    lda #1          ; Flag to turn the cursor blink back off

sta BLNSW       ; Turn off the flashing cursor

rts

October 13th 2021 – Progress on saving a sequence file

I found I had used a variable called SeqLgth that was supposed to be the number of times the joystick had been moved by the user, but I was actually using another variable called SeqPos to measure this. So after some frustrations why I couldn’t save the full sequence, all I had to do was use SeqPos times 3 for the total bytes to save to a file. Now I need to test being able to load a SEQ file and ‘re-play’ this. Note: some of the code below is from Codebase64 “Writing to a file byte-by-byte”.

; Save a recorded sequence

; This routine prompts the user for a filename (no error checking yet)

; Then saves the sequence just recorded

SaveRC  clc             ; Clear Carry flag = Set cursor position

ldy #1          ; Cursor X position

ldx #23         ; Cursor Y position

jsr plot        ; Move cursor to initial 'Filename?:' position

ldy #10

ldx #0

@loop2  lda filen,x     ; Print "Filename?:" at line 23 and 1 in from side

jsr chrout

inx

dey

bne @loop2

jsr getflnm     ; Get filename

lda filnaml     ; check if any filename actually provided

bne @cont1      ; if we got a filename let's continue

jmp SaveRC      ; let's re-prompt for a filename

@cont1  ldy #0

ldx filnaml     ; Read filnaml which will have the filename length

@loop3  lda fnseqx,y    ; Add the text ",s,w" to end of filnaml

sta filetxt,x

inx

iny

cpy #4

bne @loop3      ; Have we added all the characters yet?

clc

lda filnaml

adc #4          ; Accumulator should now contain filnaml+4

ldx #<filetxt   ; Address in memory holding filename text

ldy #>filetxt

jsr setnam      ; Kernal call to set filename

lda SeqPos

ldx #3

jsr multi

stx i_Var

sta i_Var+1

lda #$01        ; Now to add $4001

sta j_Var       ; I need to add $4001 becuase the routine below

lda #$40        ; For saving does an inc after saving each byte

sta j_Var+1     ; and therefore stops one byte short otherwise

jsr Addit       ; Add $4000 to Song Length * 4, result in k_Var

lda #$05        ; file number 5

ldx #$08        ; default to device 8

ldy #$02        ; secondary address 2

jsr setlfs      ; call SETLFS

clc

jsr open        ; call OPEN

bcs error2      ; if carry set, the file could not be opened

ldx #$05        ; filenumber 5

jsr chkout      ; call CHKOUT (file 5 now used as output)

lda #$00        ; set j_Var to start address of where in mem file

sta j_Var       ; file data starts from (being 2 bytes after $4000)

lda #$40        ; so ignore the 2 bytes that are the load address

sta j_var+1

ldy #$00

@loop4  jsr readst      ; call READST (read status byte)

bne werror      ; write error

lda (j_Var),Y   ; get byte from memory

jsr chrout      ; call CHROUT (write byte to file)

inc j_var

bne @skip4

inc j_Var+1

@skip4  lda j_Var

cmp k_Var

lda j_Var+1

sbc k_Var+1

bcc @loop4      ; next byte

close3  lda #$05        ; filenumber 5

jsr close       ; call CLOSE

jsr clrchn      ; call CLRCHN

lda #0          ; since song is now saved

sta RecFlg      ; reset the Modded flag

sta DirFlag     ; Ensure the flag to read the directory is reset

endis   rts

error1  ; Akkumulator contains BASIC error code

; most likely errors:

; A = $05 (DEVICE NOT PRESENT)

;... error handling for open errors ...

jmp close3   ; even if OPEN failed, the file has to be closed

rderror ; for further information, the drive error channel has to be read

;... error handling for read errors ...

jmp close3

error2  ; Akkumulator contains BASIC error code

; most likely errors:

; A = $05 (DEVICE NOT PRESENT)

; ... error handling for open errors ...

jmp close3    ; even if OPEN failed, the file has to be closed

werror  ; for further information, the drive error channel has to be read

; ... error handling for write errors ...

jmp close3